#

#  $Id$

#

2-28-1995 GK

In order to provide more functionality to the boot rom code I changed

Jamie's draft a little bit. All my changes have a bar sign (|) in the

79th column.

Gero Kuhlmann

===============================================================================

0. Numbering

This is Draft Net Boot Image Proposal 0.2, February 28, 1995                   |

1. Preamble - the why

Whilst researching what other boot proms do (at least those implementing

TCP/IP protocols) it is clear that each 'does their own thing' in

terms of what they expect in a boot image.

If we could all agree on working toward an open standard, O/S suppliers

and boot rom suppliers can build their products to this norm, and be confident

that they will work with each other.

This is a description of how I will implement the boot rom for

Linux. I  believe it to be flexible enough for any OS that will be loaded

when a PC boots from a network in the TCP/IP environment.

It would be good if this could be turned into some form of standard.

This is very much a first draft. I am inviting comment.

The ideas presented here should be independant of any implementation.

In the end, where there is a conflict between the final of this draft, and an

implementation, this description should prevail.

The terms I use are defined at the end.

2. The target

The target is to have a PC retrieve a boot image from a network in the TCP/IP

environment.

The boot may take place from a network adaptor rom, from a boot floppy, or

from a program in MSDOS.

3. Net Boot Process Description.

The net boot process can be started either as a result of the PC

boot process, or through normal DOS execution of a program. The net boot

program can reside on a rom, e.g. on an adaptor card, or in ram, either

as a result of reading off disk or transferred from ram.

The boot process may execute in any mode (e.g. 8086, 80386) it desires.

When it jumps to the start location in the boot image, it must be in

8086 mode and be capable of going into any mode supported by the

underlying processor.

The image cannot be loaded into address spaces below 10000h, or between

A0000h through FFFFFh, or between 98000h through 9FFFFh. Once the image

starts executing, all the memory is available to it, so it can relocate

parts of itself to these areas.

The boot process must be capable of loading the image into all other

memory locations. Specifically, where the machine supports this, this means

memory over 100000h.

The net boot process must execute the bootp protocol, followed by

the tftp protocol, as defined in the relevant rfc's.

The file name used in the tftp protocol must be that given by the bootp

record.

If less than 512 bytes are loaded, the net boot process attempts to display

on the screen any ascii data at the start of the image. The net boot

process then exits in the normal manner. For a boot prom, this will

allow normal disk booting. For DOS programs, this will mean a normal return

to DOS.

When the first 512 bytes have been loaded, the boot process checks

for an initial magic number, which is defined later. If this number

is present, the net process continues loading under the control

of the image format. The image, which is described later, tells the

net boot process where to put this record and all subsequent data.

If no initial magic number is present the net boot process checks for a second

magic number at offset 510. If the magic number 510 = 55h, 511 = AAh,

then the net process continues. If this second magic number is not

present, then the net boot process terminates the tftp protocol, displays

an error message and exits in the normal manner.

If no initial magic number is present and the second one is, the net boot

process relocates the 512 bytes to location 7c00h. The net boot process

continues to load any further image data to 10000h up. This data can overwrite

the first 512 boot bytes. If the image reaches 98000h, then any further data is

continued to be loaded above 100000h. When all the data has been loaded, the

net boot process jumps to location 0:7c00.

When the net boot process calls the image, it places 2 far pointers onto       |

the stack, in standard intel order (e.g. segment:offset representation).       |

The first far pointer which immediately follows the return address on          |

the stack, points to the loaded boot image header. The second far pointer      |

which is placed above the first one, shows to the memory area where the        |

net boot process saved the bootp reply.                                        |

4. Image Format with Initial Magic Number.

The first 512 bytes of the image file contain the image header,

and image loading information records. This contains all the

information needed by the net boot process as to where data

is to be loaded.

The magic number (in time-honoured tradition (well why not?)) is:

0 = 36h

1 = 13h

2 = 03h

3 = 1Bh

Apart from the two magic numbers, all words and double words are in PC

native endian.

Including the initial magic number the header record is:

+---------------------+

|                     |

| Initial Magic No.   |  4 bytes

+---------------------+

|                     |

| Flags and length    |  double word

+---------------------+

|                     |

| Location Address    |  double word in ds:bx format

+---------------------+

|                     |

| Execute Address     |  double word in cs:ip format

+---------------------+

The Location address is where to place the 512 bytes. The net boot

process does this before loading the rest of the image. The location

address cannot be one of the reserved locations mentioned above, but

must be an address lower than 100000h.

The rest of the image must not overwrite these initial 512 bytes, placed

at the required location. The writing of data by the net boot process

into these 512 bytes is deprecated. These 512 bytes must be available for

the image to interogate once it is loaded and running.

The execute address is the location in cs:ip of the initial instruction

once the full image has been loaded. This must be lower than 100000h,

since the initial instructions will be executed in 8086 mode. When the

jump (actaully a far call) is made to the boot image, the stack contains a

far return address, with a far pointer parameter above that, pointing

to the location of this header.

The flags and length field is broken up in the following way:

Bits 0 to 3 (lowest 4 bits) define the length of the non vendor header in

double words. Currently the value is 4.

Bits 4 to 7 define the length required by the vendor extra information

in double words. A value of zero indicates no extra vendor information.

Bits 8 to 31 are reserved for future use and must be set to zero.

After this header, and any vendor header, come the image loading information

records. These specify where data is to be loaded, how long it is, and

communicates to the loaded image what sort of data it is.

The format of each image loading information record is :

+---------------------+

| Flags, tags and     |  double word

| lengths             |

+---------------------+

|                     |

| Load Address        |  double word

+---------------------+

|                     |

| Image Length        |  double word

+---------------------+

|                     |

| Memory Length       |  double word

+---------------------+

Each image loading information record follows the previous, or the header.

The memory length, image length and load address fields are unsigned 32

numbers. They do not have the segment:offset format used by the 8086.

The flags, tags and lengths field is broken up as follows:

Bits 0 to 3 (lowest 4 bits) are the length of the non vendor part of this

header in double words. Currently this value is 4.

Bits 4 to 7 indicate the length of any vendor information, in double words.

Bits 8 to 15 are for vendor's tags. The vendor tag is a private number that

the loaded image can use to determine what sort of image is at this particular

location.

Bits 16 to 23 are for future expansion and should be set to zero.

Bits 24 to 31 are for flags, which are defined later.

Vendors may place further information after this information record, and

before the next. Each information record may have a different vendor

length.

There are two restrictions on vendor information.

One is that the header and all information records that the net boot process

is to use fall within the first 512 bytes.

The second restriction is that the net boot process must ignore all

vendor additions. The net boot process may not overwrite vendor supplied

information, or other undefined data in the initial 512 bytes.

The flags are used to modify the load address field, and to indicate

that this is the last information record that the net boot process should

use.

Bit 24 works in conjunction with bit 25 to specify the meaning of the

load address.

B24    B25

0     0    load address is an absolute 32 number

1     0    add the load address to the location one past the last byte

of the memory area required by the last image loaded.

If the first image, then add to 512 plus the location

where the 512 bytes were placed

0     1    subtract the load address from the one past the

last writeable location in memory. Thus 1 would

be the last location one could write in memory.

1     1    load address is subtracted from the start of

the last image loaded. If the first image, then

subtract from the start of where the 512 bytes were

placed

(For convenience bit 24 is byte 0 of the flag field)

Bit 26 is the end marker for the net boot process. It is set when

this is the last information record the net boot process should

look at. More records may be present, but the net boot process will not

look at them. (Vendors can continue information records out past the 512

boundary for private use in this manner).

The image length tells the net boot process how many bytes are to be loaded.

Zero is a valid value. This can be used to mark memory areas such as

shared memory for interprocessor communication, flash eproms, data in eproms.

The image length can also be different from the memory length. This allows

decompression programs to fluff up the kernel image. It also allows a file

system to be larger then the loaded file system image.

Bits 27 through 31 are not defined as yet and must be set to zero until

they are.

6. Boot prom entry points.

I have not defined boot entry points, and means of obtaining them.

It could be useful to down load part of an image, and have that image

load more of itself by using handy parts of the net boot program.

This can be considered 'for further study'.

7. Example of a boot image.

Here is an example of how the boot image would look for Linux:

0x1B031336,  /* magic number */

0x4,         /* length of header is 16 bytes, no vendor info */

0x90000000,  /* location in ds:bx format */

0x90000200,  /* execute address in cs:ip format */

/* 2048 setup.S bytes */

0x4,	       /* flags, not end, absolute address, 16 bytes this

record, no vendor info */

0x90200,     /* load address - note format */

0x800,       /* 4 8 512 byte blocks for linux */

0x800,

/* kernel image */

0x4,	       /* flags, not end, absolute address, 16 bytes this

record, no vendor info */

0x10000,     /* load address - note format */

0x80000,     /* 512K (this could be shorter */

0x80000,

/* ramdisk for root file system */

0x04000004,  /* flags = last, absolute address, 16 bytes this

record, no vendor info *//

0x100000,    /* load address - in extended memory */

0x80000,     /* 512K for instance */

0x80000,

/* Then follows linux specific information */

8. Terms

When I say 'the net boot process', I mean the act of loading the image into

memory, setting up any tables, up until the jump to the required location

in the image.

The net booting program executes the net boot process. The net boot program

may be a rom, but not neccassarily. It is a set of instructions and data

residing on the booting machine.

The image, or boot image,  consists of the data loaded by the net boot process.

When I say 'the PC boot process', I mean the general PC rom bios boot process,

the setting up of hardware, the scanning for adaptor roms, the execution

of adaptor roms, the loading in of the initial boot track. The PC boot

process will include the net boot process, if one is present.

When I say client, I mean the PC booting up.

When I say 'image host', I mean the host where the boot image is comming from.

This may not have the same architecture as the client.

The bootp protocol is defined in RFC951 and RFC1084. The tftp protocol

is defined in RFC783. These are available on many sites.

See Comer 1991 for details on how to obtain them.

A bootp server is the machine that answers the bootp request. It is not

neccassarily the image host.

'Can' and 'may' means doesn't have to, but is allowed to and might.

'Must' means just that. 'Cannot' means must not.

9 References

Comer, D.E. 1991, Internetworking with TCP/IP Vol I: Principles, Protocols,

and Architecture Second Edition, Prentice Hall, Englewood Cliffs, N.J., 1991

Stevens, W.R 1990, Unix Network Programming, Prentice Hall,

Englewood Cliffs, N.J., 1990