#

#  Cable information for the DY-4 DMV177

#

#  $Id: cable.doc,v 1.5 1998/07/18 16:56:16 joel Exp $

#

Background

==========

All serial cables described in this file were built using RJ-45 ribbon

cables which were plugged into DB-25 or DB-9 adapters wired as described.

Matching the colors of the wires in the shell is the easiest way to

make sure the cable is built properly.

The board is configured as follows:

- An MC68681 is the first two serial ports.  These are RS-232 and may

be accessed via the front panel or the P2.  The front panel connector

has both A and B serial ports on a single connector. DY-4 part number

"OBYC3ASSY901132-000 Rev-" brings these ports out to two DB-25

female connectors.

- The MC68681 serial ports are also accessible on the P2 connector.

DY-4 part number "OBYC3ASSY901080-004 Rev A" brings these out

to DB-9 female connectors.  These were not used in this effort.

- A Z85C30 is used for the third and fourth serial ports.  These are

RS-422 and may only be accessed via the P2 connector.  These are

brought out to DB-25 male connectors.  An RS-422 to RS-232 converter

was used to interface with these serial ports.

References

==========

- Linux Serial FAQ

- http://ahmed.egypt.com/electronics/serial

- DY-4 Documentation

RS-422 Converter

================

An RS-422 to RS-232 converter from B&B Electronics was used to

adapt the RS-422 signals so they could be used with a normal

RS-232 serial port.  Here is the information required to purchase

one of these:

RS-232/RS-422 Converter:     Model 422LCON

Power Supply:                Model 422PS

B&B Electronics Manufacturing Company

707 Dayton Road

PO Box 1040

Ottawa, IL 61350

Voice: (815) 433-5100

FAX:   (815)434-7094

Front Panel RS-232 Connection to a PC

=====================================

Board's DB-25    PC's DB-9     Color in

Channel A         COM1        Shell

=============   ============  =========

RX on pin 2     TX on pin 2    Red

TX on pin 3     RX on pin 3    Blue

GND on pin 7    GND on pin 5    Black

DTR on pin 20   DTR on pin 4    Brown

No other signals are used.

RS-422 Connection to a PC

=========================

This requires two cables with the converter connecting them.

P2 RS-422 to the RS-422/RS-232 Converter

========================================

Board's          Converter's    Color in

DB-25           RS-422 DB-25     Shell

===============    ==============   ========

TXD_A on pin 2     RDA on pin 5      Red

TXD_B on pin 14    RDB on pin 17     Blue

RX_A on pin 3      TDA on pin 2      Brown

RX_B on pin 16     TDB on pin 14     Green

GND on pin 7       GND on pin 7      Black

Converter's       PC's DB-9     Color in

RS-232 DB-25         COM1        Shell

===============  ==============  ========

TX on pin  2     TX on pin 2      Blue

RX on pin  3     RX on pin 3      Red

GND on pin  5    GND on pin 5      Black

DTR on pin  4    DTR on pin 4      Brown

No other signals